COLLEGE OF COMPUTING AND INFORMATION SCIENCES

DEPARTMENT OF COMPUTER SCIENCE

COURSE: BACHELOR OF SCIENCE IN COMPUTER SCIENCE

COURSE UNIT: PRACTICAL SKILLS(EMBEDDED)

GROUP 20

| NAME | GENDER | REGISTRATION NUMBER | STUDENT NO. | COLLEGE |
| --- | --- | --- | --- | --- |
| SSERUNKUUMA JOSHUA | Male | 25/U/03587/PS | 2500703587 | CoCIS |
| NEBIRAH PRECIOUS MERISAH | Female | 25/U/03541/PS | 2500703541 | CoCIS |
| TAMBWE RAHIM STONE | Male | 25/E/03597/EVE | 2500703597 | CoCIS |
| MUGABE GIDEON | Male | 25/U/03455/PS | 2500703455 | CoCIS |
| NANSUBUGA ZAHRA | Female | 25/U/03531/PS | 2500703531 | Jinja |

Smart IV Fluid Monitoring System

Abstract

## Problem Statement

Hospitals and health centres, particularly in resource-limited settings, face persistent challenges in monitoring intravenous (IV) fluid therapy. Nurses must move frequently between patients to visually check fluid levels, which consumes valuable time, and a single nurse is often responsible for many patients at once, making continuous monitoring difficult. Because conventional IV stands provide no early warning, an IV bag can run empty before staff notice, interrupting treatment and increasing the risk of air entering the line. This lack of real-time visibility into IV status creates unnecessary patient safety risks and leaves nurses performing repetitive manual checks instead of attending to other critical responsibilities.

## Proposed Solution and Methodology

This project presents a Smart IV Fluid Monitoring System that automatically tracks IV fluid levels and alerts healthcare workers when replacement is required. A weight or fluid-level sensor mounted on the IV stand continuously reports data to an Arduino microcontroller, which classifies the fluid level as Normal, Low, or Critical. As the level drops, colour-coded LED indicators (green, yellow, red) and a liquid crystal display (LCD) communicate status at a glance, while a buzzer sounds a short audio alert once the critical threshold is reached so that immediate attention is not missed. After the bag is replaced, a nurse presses a reset button that recalibrates the sensor and resumes automatic monitoring. The system was implemented as a working prototype using low-cost Arduino hardware, requiring no mobile application or internet connectivity, with all sensing and alert logic encoded directly in the Arduino sketch.

## Results

Testing confirmed that the prototype correctly distinguishes between Normal, Low, and Critical IV fluid levels and triggers the corresponding visual and audio alerts in real time, giving nurses adequate warning to prepare a replacement bag before treatment is interrupted. The reset function was verified to reliably recalibrate the sensor and resume monitoring after a bag change. Although built on inexpensive, single-station hardware, the underlying sensing and alert logic is hardware-independent and can be scaled to a centralised, multi-bed hospital ward by networking multiple monitoring units to a shared alert dashboard.

## Conclusion

This work demonstrates that the core contribution of the project lies not in the hardware itself, but in the sensing and alert logic that governs how fluid levels are classified, displayed, and escalated to staff. The Smart IV Fluid Monitoring System provides an affordable, reliable, and easy-to-operate solution for reducing unnoticed IV depletion, easing nurse workload, and improving patient safety, illustrating how simple, low-cost embedded systems can meaningfully strengthen healthcare delivery in Uganda and other resource-limited settings.